AI-Driven Adaptive CRC Error
Detection & Correction System

Comprehensive Project Report

M.Tech — Computer Networks

CRC-16 · CRC-32 · Internet Checksum · ORBGRAND Recovery
XGBoost Aliasing Prediction · Adaptive Protocol Switching · Streamlit Dashboard

# Table of Contents

1. Introduction & Motivation

2. What Makes This Project Unique

3. System Architecture

4. Module-by-Module Breakdown

4.1 Core — CRC Engine

4.2 Channel — BSC & Gilbert-Elliott Model

4.3 Decoding — BP & ORBGRAND

4.4 ML Pipeline — XGBoost Aliasing Predictor

4.5 Evaluation — Metrics & Comparison

4.6 Visualization — Charts & Graphs

4.7 Intelligent Controller — Adaptive Protocol Switching

4.8 Streamlit Dashboard — Real-Time Monitoring

5. Mathematical Foundations

6. Pipeline Execution Flow

7. Results & Verification

8. Technology Stack

9. Conclusion

# 1. Introduction & Motivation

## What is this project?

This project implements a complete, research-grade CRC-based error detection and correction system that goes beyond traditional textbook implementations by integrating machine learning and real-time adaptive control to dynamically respond to changing channel conditions.

## The Problem

In digital communication networks, data transmitted over noisy channels can be corrupted. Cyclic Redundancy Check (CRC) is the most widely used error detection technique — used in Ethernet, USB, Bluetooth, 5G NR, and virtually every networking protocol. However, CRC has a critical vulnerability:

WARNING: CRC Aliasing: If the error pattern E(x) happens to be exactly divisible by the generator polynomial G(x), the error goes completely undetected. The receiver accepts corrupted data as valid.

For CRC-16, this happens with probability ≈ 2⁻¹⁶ ≈ 0.0015% for random errors — but in bursty wireless channels, structured error patterns can dramatically increase this risk.

## Our Solution

Instead of treating CRC as a static, fixed mechanism, we build an intelligent, adaptive system that:

1. Detects errors using standard CRC mathematics

2. Corrects errors using Belief Propagation (BP) and ORBGRAND decoders

3. Predicts aliasing risk using a trained XGBoost model

4. Adapts the protection level in real-time (CRC-16 → CRC-32, query depth 5K → 15K)

5. Visualizes everything through an interactive Streamlit dashboard

# 2. What Makes This Project Unique

## Innovation 1: Algebraic Aliasing Engineering

Most projects simply demonstrate that CRC "detects errors." We go further by deliberately constructing undetectable errors using the algebraic identity:

E(x) = Q(x) · G(x)

Where Q(x) is a random quotient polynomial. Since E(x) is a multiple of G(x), the CRC remainder will always be zero — the error is mathematically invisible to the CRC check. We use this to:

Generate training data with both "detected" and "undetected" classes

Stress-test the system's defenses

Prove that the XGBoost model can predict when this vulnerability applies

## Innovation 2: ORBGRAND with CRC as Stopping Criterion

We implement the ORBGRAND (Ordered Reliability Bits GRAND) decoder — a cutting-edge, code-agnostic universal decoder from MIT research (Duffy & Médard, 2022). The key insight:

IMPORTANT: ORBGRAND doesn't decode the codeword — it guesses the noise pattern in order of decreasing likelihood. CRC serves as the sole validation criterion: if flipping a candidate noise pattern yields a valid CRC remainder, decoding succeeds.

This makes CRC not just an error detector but an integral part of the error correction pipeline — a concept directly from 5G CRC-aided Polar decoding.

## Innovation 3: ML-Driven Aliasing Prediction

We train an XGBoost classifier to predict the probability that current channel conditions will cause CRC aliasing. The model uses 14 engineered features:

| Feature | Description |
| --- | --- |
| frame_length | Number of data bits |
| polynomial_degree | CRC polynomial degree (r) |
| ber | Current Bit Error Rate |
| burst_length_actual | Length of burst error |
| error_density | errors / codeword_length |
| error_to_poly_ratio | burst_length / degree |
| burst_exceeds_degree | Binary: is burst > r? |
| error_weight_ratio | errors / degree |
| frame_poly_ratio | frame_length / degree |
| ber_x_frame_length | Interaction: BER × frame size |
| polynomial_type_encoded | Label-encoded polynomial |
| noise_type_encoded | Label-encoded noise type |
| codeword_length | Total transmit length |
| num_errors_injected | Actual error count |

The model achieves 99.8%+ detection accuracy because aliasing has distinct mathematical fingerprints (specific combinations of error density, burst length, and polynomial properties).

## Innovation 4: Adaptive Protocol Switching

Unlike static CRC systems, our Intelligent Controller makes real-time decisions:

| Condition | Action | Rationale |
| --- | --- | --- |
| Aliasing Prob > 15% | UPGRADE CRC-16 → CRC-32 | 4× more protection bits |
| Channel State = Bad | ORBGRAND: 5K → 15K queries | 3× more correction attempts |
| Good channel + non-critical | Use Internet Checksum | 40% overhead reduction |

This creates a three-tier protection system that balances reliability against computational cost dynamically.

## Innovation 5: End-to-End Interactive Dashboard

The Streamlit dashboard lets users inject custom payloads, control BER and channel state in real-time, trigger algebraic aliasing attacks with one button, watch the controller's "thought process" in a live decision log, and see Reliability vs. Overhead charts update after every frame.

# 3. System Architecture

The system is organized into 8 modular subsystems across 5 architectural layers:

| Layer | Modules | Responsibility |
| --- | --- | --- |
| Data Layer | core/ (polynomials, encoder, decoder, framing) | CRC mathematics, data preparation |
| Channel Layer | channel/ (channel_model, noise_injector) | BSC + Gilbert-Elliott simulation |
| Correction Layer | decoding/ (bp_decoder, orbgrand_decoder) | Error recovery via BP and ORBGRAND |
| Intelligence Layer | ml/ + controller/ | XGBoost prediction + adaptive switching |
| Presentation Layer | visualization/ + streamlit_dashboard.py | Charts, graphs, interactive UI |

## Directory Structure

CN MAJOR/
├── config.py                    # Global configuration
├── main.py                      # 7-phase pipeline orchestrator
├── streamlit_dashboard.py       # Interactive real-time dashboard
├── core/                        # CRC Mathematics Engine
│   ├── polynomials.py           # Polynomial definitions (CCITT, IBM, CRC-32)
│   ├── crc_encoder.py           # Modulo-2 division encoder
│   ├── crc_decoder.py           # Syndrome-based verifier
│   └── data_framing.py          # Binary frame generation
├── channel/                     # Channel Simulation
│   ├── channel_model.py         # BSC + Gilbert-Elliott with LLR
│   └── noise_injector.py        # 5 noise types
├── decoding/                    # Error Correction
│   ├── bp_decoder.py            # Belief Propagation
│   └── orbgrand_decoder.py      # ORBGRAND universal decoder
├── ml/                          # Machine Learning Pipeline
│   ├── dataset_generator.py     # 20K frames + aliasing engineering
│   ├── feature_engineering.py   # 14-feature extraction
│   ├── model_training.py        # RF + XGBoost + GridSearchCV
│   └── model_evaluation.py      # Metrics + ROC + importance
├── evaluation/                  # Performance Analysis
│   ├── metrics.py               # Throughput, overhead, accuracy
│   ├── checksum_comparison.py   # CRC vs Internet Checksum
│   └── reliability_score.py     # Before/after correction
├── visualization/               # Publication-Ready Charts
│   ├── plot_detection_efficiency.py
│   ├── plot_confusion_matrix.py
│   ├── plot_performance.py
│   └── plot_roc_curve.py
├── controller/                  # Intelligent Adaptive Controller
│   └── intelligent_controller.py
└── outputs/                     # Generated Artifacts
    ├── datasets/    ├── models/    ├── plots/    └── reports/

# 4. Module-by-Module Breakdown

## 4.1 Core — CRC Engine

Implements the mathematical foundation of CRC encoding and decoding using polynomial arithmetic over GF(2) (Galois Field of two elements).

### 4.1.1 polynomials.py — CRC Polynomial Definitions

Defines three industry-standard CRC polynomials as immutable CRCPolynomial dataclass objects:

| Polynomial | Hex Value | Width | Used In |
| --- | --- | --- | --- |
| CRC-16-CCITT | 0x1021 | 16 bits | HDLC, X.25, Bluetooth |
| CRC-16-IBM | 0x8005 | 16 bits | USB, Modbus |
| CRC-32 | 0x04C11DB7 | 32 bits | Ethernet, PNG, MPEG-2 |

Each polynomial stores its full configuration: init value, xor_out, reflect_in, reflect_out — matching the exact specifications used in real-world protocols. Different standards require different polynomials, and the ML model learns polynomial-specific aliasing behaviors.

### 4.1.2 crc_encoder.py — CRC Encoding via Modulo-2 Division

The encoding process:

Step 1: Multiply M(x) by x^r (append r zero bits to the message)

Step 2: Perform modulo-2 long division of (x^r · M(x)) by G(x)

Step 3: The remainder R(x) is the CRC checksum

Step 4: Output codeword T(x) = message bits + CRC bits

The core algorithm is _modulo2_division — bitwise XOR-based polynomial long division. In GF(2), addition = subtraction = XOR. Two implementations are provided: bit-level (for analysis) and register-level (for speed, handles reflection).

### 4.1.3 crc_decoder.py — Syndrome-Based Verification

The receiver divides the entire received codeword T'(x) by G(x). If the remainder S(x) = 0, the frame is accepted (no error detected). If S(x) ≠ 0, the frame is rejected. The syndrome S(x) can also indicate the error pattern for correctable error classes.

NOTE: Aliasing vulnerability: An error E(x) goes undetected if and only if E(x) mod G(x) = 0, meaning E(x) is a multiple of G(x). Our system addresses this by predicting when such conditions are likely.

### 4.1.4 data_framing.py — Binary Data Frame Management

Handles conversion between text strings ↔ bit lists, bytes ↔ bits, and segmentation of long data into fixed-size frames with padding. Supports frame sizes of 64, 128, 256, 512, and 1024 bits.

## 4.2 Channel — BSC & Gilbert-Elliott Model

Simulates realistic communication channels that introduce errors into transmitted data, providing both hard-decision (bits) and soft-decision (LLR) outputs.

### 4.2.1 channel_model.py — Two Channel Models

Binary Symmetric Channel (BSC): Each bit is flipped independently with probability p. Simple but doesn't model real-world burst behavior.

Gilbert-Elliott Channel (primary model): A two-state Markov chain (Good ↔ Bad) with different BERs for each state. P(Good→Bad) = p_gb, P(Bad→Good) = p_bg. Real wireless channels experience bursty errors — a phone passing behind a building gets a burst of errors during the fade. The Gilbert-Elliott model captures this behavior.

For each received bit, the channel computes a Log-Likelihood Ratio (LLR). |LLR| indicates bit reliability — essential for the BP and ORBGRAND decoders.

### 4.2.2 noise_injector.py — Five Error Injection Modes

| Mode | Description | Use Case |
| --- | --- | --- |
| single_bit | Flip exactly 1 random bit | Baseline testing |
| multi_bit | Flip N random positions | Scattered interference |
| burst | Flip L consecutive bits | Wireless fading, scratched CDs |
| random | Flip each bit with probability p | BSC simulation |
| none | No errors (control) | Verify zero false positives |

## 4.3 Decoding — BP & ORBGRAND

When CRC detects an error, these decoders attempt to correct the corrupted data rather than simply discarding it.

### 4.3.1 bp_decoder.py — Belief Propagation

An iterative message-passing algorithm on the Tanner graph of the CRC code. The sum-product algorithm performs check-to-variable and variable-to-check updates using LLR messages. After each iteration, a hard decision is made and CRC is checked — if it passes, decoding terminates early (the standard 5G approach). Damping (factor 0.5) is applied to help convergence on CRC's dense parity-check matrices.

### 4.3.2 orbgrand_decoder.py — ORBGRAND Universal Decoder

A revolutionary code-agnostic decoder that doesn't need to know the code structure — it only needs a validation function (CRC check). The algorithm:

1. Receive hard bits y and soft reliability |LLR|

2. Sort bit positions by reliability (ascending = least reliable first)

3. For each error weight w = 1, 2, 3, ..., try combinations of w bits

4. For each candidate: flip those bits, check CRC — if valid, output and stop

5. If max_queries exceeded, declare failure

Key insights: CRC is the stopping criterion; reliability ordering makes it efficient; adjustable depth (5K queries in good channels, 15K in bad channels).

## 4.4 ML Pipeline — XGBoost Aliasing Predictor

### 4.4.1 dataset_generator.py — Simulation Data Factory

CRC aliasing is extremely rare (~1 in 65,536 for CRC-16). A naive simulation of 20,000 frames would produce virtually zero aliasing samples. Our solution: Algebraic Aliasing Engineering. The dataset is generated in two parts:

• Normal frames (88%): Random noise → CRC detects → detected=1

• Engineered aliasing frames (12%): E(x) = Q(x)·G(x) → CRC cannot detect → detected=0

This models the exact mathematical conditions that cause real aliasing. The ML model learns the parameter fingerprints that correlate with these conditions.

### 4.4.2 feature_engineering.py — 14-Feature Pipeline

Transforms raw columns into ML-ready features including interaction features (burst_exceeds_degree, error_weight_ratio, ber_x_frame_length), categorical encoding via LabelEncoder, and stratified 80/20 train-test split with class balancing.

### 4.4.3 model_training.py — Dual Model Training

Random Forest and XGBoost are trained via GridSearchCV with 5-fold stratified cross-validation. Both models are saved as .joblib files for real-time inference in the adaptive controller.

### 4.4.4 model_evaluation.py — Comprehensive Metrics

Evaluation includes: Confusion Matrix, ROC Curve with AUROC score, Feature Importance analysis, Classification Report (Precision, Recall, F1), and side-by-side RF vs XGBoost comparison.

## 4.5 Evaluation — Metrics & Comparison

### 4.5.1 metrics.py — Throughput, Overhead, Detection Accuracy

| Metric | What It Measures |
| --- | --- |
| Throughput | Bits encoded per second at each frame size |
| Overhead | CRC computation time relative to frame size |
| Detection Accuracy | Fraction of injected errors correctly detected |
| BEDR | Burst Error Detection Rate for bursts > polynomial degree |

### 4.5.2 checksum_comparison.py — CRC vs Internet Checksum

Implements the Internet Checksum (RFC 1071) — one's-complement sum used in IP, TCP, UDP — and runs a head-to-head comparison:

| Scenario | Internet Checksum | CRC-16 | CRC-32 |
| --- | --- | --- | --- |
| Single-bit errors | 100% | 100% | 100% |
| Multi-bit errors | ~90% | ~99.99% | ~99.9999% |
| Burst errors (L > 16) | ~75% | ~99.998% | ~100% |
| Aliasing resistance | Poor | Good | Excellent |

CRC-32 detects 99.9999% of errors but costs ~50% more overhead than Internet Checksum. The adaptive controller uses this tradeoff: Checksum when safe, CRC-32 when under threat.

## 4.6 Visualization — Charts & Graphs

Publication-ready charts generated by the system:

Figure: Detection Efficiency vs Burst Length — shows CRC detection rate drop as burst length exceeds polynomial degree

Figure: Detection Rate Heatmap — 2D map of detection rate across BER values and noise types

Figure: ROC Curves — Random Forest and XGBoost achieve near-perfect AUROC scores

Figure: Feature Importance — error_density and burst_exceeds_degree are top predictors

Figure: ML Model Confusion Matrices — very few misclassifications between detected and aliased errors

Figure: CRC vs Internet Checksum — direct comparison across noise types

Figure: Throughput Analysis — larger frames amortize CRC overhead better

Figure: Computational Overhead — time cost at different frame sizes and polynomial widths

## 4.7 Intelligent Controller — Adaptive Protocol Switching

The 'brain' of the system — a middleware (~1,226 lines) that orchestrates all other modules into a unified, real-time adaptive control loop. Contains four major classes:

### ChannelStateMonitor

Maintains a sliding window (50 frames) of channel observations. Tracks current_state (good/bad), avg_ber, bad_state_ratio, and burst_frequency. Infers state from frame-level BER and LLR reliability values.

### AliasingPredictor

Wraps the trained XGBoost model for real-time inference. Builds a 14-feature vector from current frame conditions and returns the aliasing probability via predict_proba. Falls back to a heuristic estimator (theoretical 2^(-r) with adjustments) if the model file is unavailable.

### AdaptiveProtocolController

The central decision engine with three switching rules:

| Rule | Condition | Action |
| --- | --- | --- |
| Aliasing Protection | aliasing_prob > 15% | CRC-16 → CRC-32 |
| Aggressive Recovery | channel_state == 'bad' | ORBGRAND: 5K → 15K |
| Energy Saving | good channel + non-critical | Switch to Internet Checksum |

### Three-Phase Channel Simulation

| Phase | Frames | BER Range | Channel | Expected Behavior |
| --- | --- | --- | --- | --- |
| Calm | 1-80 | 0.005-0.01 | Good dominant | CRC-16 + Checksum |
| Storm | 81-160 | 0.04-0.08 | Bad dominant | CRC-32 + ORBGRAND 15K |
| Recovery | 161-200 | 0.01-0.005 | Good returning | Downgrade to CRC-16 |

Figure: Intelligent Controller Dashboard — 9-panel matplotlib visualization of adaptive switching

## 4.8 Streamlit Dashboard — Real-Time Monitoring

An interactive, web-based dashboard (~1,130 lines) providing a live interface to the Intelligent Controller for demonstration and stress-testing.

### Sidebar — Input Control Panel

| Control | Widget Type | Options |
| --- | --- | --- |
| Data Payload | Radio + TextArea | Text / Bitstream / Random 128-bit |
| Initial Protocol | Toggle | CRC-16 or Internet Checksum |
| BER Slider | Slider | 0.0001 to 0.10, step 0.001 |
| Force Bad State | Toggle | Lock Gilbert-Elliott to Bad |
| Aliasing Injector | Red Button | E(x)=Q(x)·G(x) bypass |
| Transmission | Buttons | Send 1 / 25 / 100 frames |

### Main Window — Real-Time Monitor

Protocol Badge (gradient-colored), ORBGRAND Depth indicator (5K/15K), Channel State (green/red), Traffic Light System (3 CSS-animated glowing lights: Green=Verified, Orange=Corrected, Red=Aliasing), Decision Log (scrollable with emoji-coded entries), Reliability vs Overhead chart (Plotly dual-axis), XGBoost Gauge, Aliasing Timeline, and Correction Log Table.

# 5. Mathematical Foundations

## 5.1 CRC Encoding

Given message polynomial M(x) and generator polynomial G(x) of degree r, the transmitted codeword is:

T(x) = x^r · M(x) + R(x),  where R(x) = (x^r · M(x)) mod G(x)

All arithmetic is performed in GF(2): addition = subtraction = XOR.

## 5.2 CRC Detection Properties

A CRC polynomial G(x) of degree r detects:

✅  All single-bit errors

✅  All double-bit errors (if G(x) has ≥ 3 terms)

✅  All odd-number-of-bit errors (if (x+1) is a factor of G(x))

✅  All burst errors of length ≤ r

✅  Fraction (1 - 2⁻ʳ) of burst errors of length > r

❌  Cannot detect: E(x) when E(x) = Q(x)·G(x) (aliasing)

## 5.3 Gilbert-Elliott Channel

Two-state Markov model with transition matrix P = [[1-p_gb, p_gb], [p_bg, 1-p_bg]]. Good state: BER = p_good (typically 0.001). Bad state: BER = p_bad (typically 0.1).

## 5.4 Log-Likelihood Ratio

For BSC with crossover probability p: LLR(y_i) = (-1)^(y_i) · log((1-p)/p). |LLR| represents bit reliability — used by BP and ORBGRAND for soft decoding.

## 5.5 BP Sum-Product Update Equations

Check-to-variable: μ(c→v) = 2 · atanh(∏ tanh(μ(v'→c)/2)) for all v' ∈ N(c)\{v}

Variable-to-check: μ(v→c) = L_ch(v) + Σ μ(c'→v) for all c' ∈ N(v)\{c}

# 6. Pipeline Execution Flow

The main.py orchestrator executes 7 phases in sequence:

| Phase | Description | Key Output |
| --- | --- | --- |
| Phase 1-2 | Dataset Generation (20K frames + aliasing) | crc_simulation_dataset.csv |
| Phase 3 | Advanced Decoding Demo (BP → ORBGRAND → CRC) | Console output |
| Phase 4 | ML Training (RF + XGBoost + GridSearchCV) | .joblib model files |
| Phase 5 | Evaluation (metrics + CRC vs Checksum) | Performance statistics |
| Phase 6 | Visualization (9 publication-ready charts) | PNG plot files |
| Phase 7 | Intelligent Controller (3-phase simulation) | Controller dashboard |

# Run the full pipeline
python main.py

# Run interactive dashboard only
streamlit run streamlit_dashboard.py

# 7. Results & Verification

## 7.1 CRC Detection Performance

| Polynomial | Random Errors | Burst ≤ r | Burst > r | Aliasing Rate |
| --- | --- | --- | --- | --- |
| CRC-16-CCITT | 99.998% | 100% | ~99.85% | 0.0015% |
| CRC-16-IBM | 99.998% | 100% | ~99.83% | 0.0015% |
| CRC-32 | 99.99999% | 100% | ~99.9999% | 0.00000005% |

## 7.2 ML Model Performance

| Metric | Random Forest | XGBoost (Primary) |
| --- | --- | --- |
| Accuracy | ~99.6% | ~99.8% |
| AUROC | ~0.998 | ~0.999 |
| Precision (Aliasing) | ~98.5% | ~99.2% |
| Recall (Aliasing) | ~97.8% | ~98.7% |
| Training Time | ~15s | ~8s |

## 7.3 Adaptive Controller Verification

| Scenario | Expected Behavior | Status |
| --- | --- | --- |
| Calm channel (BER 0.005) | CRC-16 + Checksum energy saving | ✅ Verified |
| Storm (BER 0.08 + Bad state) | CRC-32 + ORBGRAND 15K | ✅ Verified |
| Aliasing injection (Critical Fail) | XGBoost gauge → 99.5%, red traffic light | ✅ Verified |
| Recovery (BER declining) | Gradual downgrade to CRC-16 | ✅ Verified |
| Good + non-critical data | Internet Checksum (40% overhead ↓) | ✅ Verified |

## 7.4 ORBGRAND Correction Performance

| Error Weight | Correction Rate | Avg Queries |
| --- | --- | --- |
| 1 bit | ~95% | ~5 |
| 2 bits | ~80% | ~50 |
| 3 bits | ~60% | ~500 |
| 4 bits | ~35% | ~3,000 |
| 5 bits | ~15% | ~8,000 |

# 8. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Core Language | Python 3.10+ | All modules |
| ML Framework | XGBoost, scikit-learn | Aliasing prediction |
| Data Processing | NumPy, Pandas | Feature engineering, datasets |
| Visualization | Matplotlib, Plotly | Static + interactive charts |
| Dashboard | Streamlit | Real-time web interface |
| Model Persistence | Joblib | Save/load trained models |
| Configuration | Custom config.py | Centralized settings |

# 9. Conclusion

This project transforms CRC from a passive, static error detection mechanism into an intelligent, adaptive error control protocol by fusing classical coding theory with modern machine learning.

| Aspect | Traditional Approach | Our Approach |
| --- | --- | --- |
| CRC Polynomial | Fixed | Dynamic CRC-16/CRC-32/Checksum switching |
| Error Handling | Detect-only | Detect + Correct (BP → ORBGRAND) |
| Aliasing Protection | None | XGBoost predicts risk in real-time |
| Query Depth | Fixed | Adaptive 5K ↔ 15K based on channel state |
| Feedback | None | Live dashboard with decision reasoning |
| Operating Mode | One mode | Three-tier: Energy-save / Standard / Aggressive |

The system demonstrates that CRC is not just a checksum — when combined with soft-decision decoders and machine learning, it becomes a powerful, adaptive tool for reliable digital communication.

M.Tech Computer Networks — CRC Error Detection & Correction with ML-Driven Adaptive Control