Full Working of the Streamlit
Deployed Model

Step-by-Step Execution Trace
From Button Click to Dashboard Update

AI-Driven Adaptive CRC Error Control System
M.Tech — Computer Networks

# Overview: What Exactly is Running?

The Streamlit dashboard is not just a simple visualization tool — it is running a complete, live simulation of a digital communication system. Every time you click "Send Frame," the following happens in real-time:

Your Data → CRC Encode → Noisy Channel → CRC Check → Error Correction → ML Prediction → Dashboard Update

The "model" deployed is actually three models working together:

| Component | Type | Role |
| --- | --- | --- |
| XGBoost Classifier | Machine Learning | Predicts aliasing probability from channel conditions |
| Belief Propagation (BP) | Iterative Decoder | Corrects errors using soft LLR information |
| ORBGRAND | Universal Decoder | Noise-guessing decoder (fallback if BP fails) |

# Phase 0: Initialization (happens once when page opens)

## Step 0.1 — Load the Pre-Trained XGBoost Model

When you first open http://localhost:8501, the function init_session_state() runs. It loads the XGBoost classifier from outputs/models/xgboost_model.joblib. This model was trained on 20,000 simulated frames (88% normal + 12% algebraically-engineered aliasing errors). The model learned which combinations of BER, burst length, error density, and polynomial degree are most likely to cause CRC aliasing.

model_path = "outputs/models/xgboost_model.joblib"
xgb_model = joblib.load(model_path)

## Step 0.2 — Create the Three Controller Components

monitor   = ChannelStateMonitor(window_size=50)    # Tracks Good/Bad channel state
predictor = AliasingPredictor(model=xgb_model)     # Wraps XGBoost for live inference
controller = AdaptiveProtocolController(monitor, predictor)  # Decision engine
channel   = ChannelModel(seed=42)                  # Gilbert-Elliott channel simulator

## Step 0.3 — Initialize All History Buffers

Empty lists created for: decision_log, reliability_history, overhead_history, aliasing_prob_history, state_timeline, crc_mode_timeline, correction_log, and counters for every statistic (total frames, CRC-32 upgrades, energy saves, etc.).

NOTE: All of this persists across button clicks via st.session_state — Streamlit remembers everything until you click 'Reset Session.'

# Phase 1: User Input (Sidebar Controls)

When you interact with the sidebar, you set 5 parameters that control the simulation:

## 1.1 — Data Payload

| Input Mode | What Happens | Example |
| --- | --- | --- |
| Text Message | Each character → 8 bits (ASCII) | "Hello" → 01001000 01100101 ... |
| Custom Bitstream | Your binary string parsed directly | 10110100 → [1,0,1,1,0,1,0,0] |
| Random (128-bit) | System generates 128 random 0s and 1s | [0,1,1,0,1,0,0,1,...] |

## 1.2 — Initial Protocol Toggle

OFF (default): Start with CRC-16 — the system may upgrade to CRC-32 if needed. ON: Start with Internet Checksum — lighter protection, the system will upgrade only if danger is detected.

## 1.3 — BER (Bit Error Rate) Slider: 0.0001 to 0.10

| BER Value | Meaning | Expected Errors per 128-bit frame |
| --- | --- | --- |
| 0.001 | Clean channel | ~0.13 (rare errors) |
| 0.01 | Moderate noise | ~1.3 errors per frame |
| 0.05 | Heavy noise | ~6.4 errors per frame |
| 0.10 | Extremely noisy | ~12.8 errors per frame |

## 1.4 — Force Gilbert-Elliott 'Bad' State

OFF: Channel naturally transitions between Good ↔ Bad states. ON: Forces the channel to stay in the Bad state permanently (higher error rate, bursty errors, 10× the BER, transitions biased towards staying Bad).

## 1.5 — 'INJECT CRITICAL FAIL' Button

This triggers algebraic aliasing engineering — constructs a special error pattern E(x) = Q(x) · G(x) that is mathematically guaranteed to bypass CRC detection. Since E(x) is a multiple of the generator polynomial G(x), dividing by G(x) yields remainder = 0. The CRC check passes even though the data is corrupted.

# Phase 2: Frame Processing (the core engine)

When you click "Send 1 Frame" (or 25/100), the function process_single_frame() executes. Here's exactly what happens, step by step:

## Step 2.1 — Estimate Channel Conditions

The system estimates how many errors the channel will likely introduce based on the BER you set:

noise_type = "burst" if force_bad_state else "random"
num_errors_est = max(1, int(ber * frame_len))        # e.g., 0.01 × 128 = 1
burst_length_est = max(1, int(ber * frame_len * 2))  # e.g., 0.01 × 128 × 2 = 2

## Step 2.2 — XGBoost Aliasing Prediction (THE ML MODEL)

This is where the trained XGBoost model runs inference. The predictor:

### Step A — Build a 14-dimensional feature vector:

| # | Feature Name | Calculation | Example Value |
| --- | --- | --- | --- |
| 1 | frame_length | Length of user's data | 128 |
| 2 | codeword_length | frame_length + polynomial_degree | 144 |
| 3 | polynomial_degree | Degree of G(x) | 16 |
| 4 | polynomial_type_encoded | CCITT=0, IBM=1, CRC-32=2 | 0 |
| 5 | noise_type_encoded | burst=0, multi=1, random=2, single=3 | 2 |
| 6 | ber | User's BER slider value | 0.01 |
| 7 | burst_length_actual | Estimated burst length | 2 |
| 8 | num_errors_injected | Estimated error count | 1 |
| 9 | error_density | errors / codeword_length | 0.0069 |
| 10 | error_to_poly_ratio | burst_length / degree | 0.125 |
| 11 | burst_exceeds_degree | 1 if burst > degree, else 0 | 0 |
| 12 | error_weight_ratio | min(errors / degree, 10) | 0.0625 |
| 13 | frame_poly_ratio | frame_length / degree | 8.0 |
| 14 | ber_x_frame_length | BER × frame_length | 1.28 |

### Step B — Feed to XGBoost's predict_proba():

proba = self.model.predict_proba(features)   # Returns [[P(undetected), P(detected)]]
aliasing_prob = float(proba[0, 0])           # e.g., 0.004 (0.4%)

The model outputs two probabilities: proba[0,0] = probability the error will be undetected (aliasing), and proba[0,1] = probability the error will be detected. We use the aliasing probability to make the switching decision.

### Example XGBoost Predictions:

| BER | Burst Length | Error Density | XGBoost Output | Action |
| --- | --- | --- | --- | --- |
| 0.001 | 2 | 0.007 | 0.4% (safe) | Stay on CRC-16 |
| 0.05 | 20 | 0.08 | 22% (dangerous) | UPGRADE to CRC-32! |
| 0.10 | 40 | 0.15 | 99.5% (critical) | CRC-32 + ORBGRAND 15K |

## Step 2.3 — Adaptive Switching Decision

Based on the XGBoost prediction, the controller decides which protection to use:

| Condition | Action Taken | Reason Logged |
| --- | --- | --- |
| aliasing_prob > 15% | Mode = CRC-32 | ⚠️ Aliasing Prob X% > 15% → UPGRADING to CRC-32 |
| Checksum mode + good + safe | Mode = Internet Checksum | ⚡ Energy-Efficiency: overhead ↓ 40% |
| Otherwise (normal) | Mode = CRC-16 | ✅ Standard CRC-16 (aliasing prob = X%) |
| channel_state == 'bad' | ORBGRAND: 5K → 15K | 🔄 Bad channel → ORBGRAND ↑ 15,000 |

## Step 2.4 — CRC Encoding

The message bits are encoded with the chosen protection:

| Mode | Polynomial | Message → Codeword | Overhead |
| --- | --- | --- | --- |
| CRC-32 | G(x) = 0x04C11DB7, degree 32 | 128 bits → 160 bits | 1.5× (high) |
| CRC-16 | G(x) = 0x1021, degree 16 | 128 bits → 144 bits | 1.0× (standard) |
| Checksum | RFC 1071 one's complement | 128 bits → 144 bits | 0.6× (low) |

Inside CRC encode():

1. Multiply M(x) by x^r — append r zero bits to the message

2. Modulo-2 long division — divide by G(x) using XOR operations

3. The remainder R(x) = the CRC checksum

4. Output: T(x) = message_bits + R(x) — ensures T(x) mod G(x) = 0

## Step 2.5 — Channel Transmission (Gilbert-Elliott)

The encoded codeword is transmitted through a simulated noisy channel:

received, llr, error_pattern = channel.gilbert_elliott_transmit(
    codeword,
    p_good = ber,         # Error rate in Good state
    p_bad  = ber * 10,    # Error rate in Bad state (10× worse)
    p_gb   = 0.05,        # Probability of Good → Bad transition
    p_bg   = 0.5,         # Probability of Bad → Good transition
)

What happens inside the channel, for each bit position i:

1. Check current state (Good or Bad)

2. Flip the bit with probability p_good or p_bad

3. Compute LLR: log((1-p)/p) if received=0, else -log((1-p)/p)

4. Randomly transition state: Good→Bad with p_gb, Bad→Good with p_bg

Output:

| Output | Type | Description |
| --- | --- | --- |
| received | List[int] | The corrupted codeword (some bits flipped) |
| llr | List[float] | Soft reliability per bit (how confident we are) |
| error_pattern | List[int] | Which bits were actually flipped (for analysis) |

## Step 2.6 — Aliasing Injection (only if CRITICAL FAIL pressed)

If you pressed the red button, the system constructs a mathematically undetectable error:

# Construct E(x) = Q(x) · G(x)
# 1. Take the polynomial G(x) = 0x1021 as bit list
# 2. Choose random quotient Q(x)
# 3. Multiply: E(x) = Q(x) × G(x) in GF(2) — XOR-based multiplication
# 4. XOR the error onto the received codeword
for k in range(len(received)):
    received[k] ^= alias_error[k]

Since E(x) is a multiple of G(x), the CRC check computes: T'(x) mod G(x) = (T(x) ⊕ E(x)) mod G(x) = 0 ⊕ 0 = 0. The error is invisible to CRC.

## Step 2.7 — Channel State Monitoring

The error pattern is fed to the ChannelStateMonitor's sliding window (last 50 frames). It counts errors, computes moving-average BER, and if BER > 5% or LLR values indicate low reliability, the state is classified as 'bad'. Otherwise 'good'.

## Step 2.8 — CRC Verification & Error Correction (THE DECODERS)

This is the detection and correction pipeline:

Detection & Correction Cascade:

received codeword
                          │
                    ┌─────▼─────┐
                    │ CRC Check │  (divide by G(x), check remainder)
                    └─────┬─────┘
                          │
              ┌───────────┼───────────┐
              │           │           │
         Remainder=0  Remainder=0  Remainder≠0
         & no errors  & errors      │
              │         exist        │
              ▼           ▼          ▼
           ✅ PASS    💀 ALIASING   Error Detected!
                    (undetected     │
                     error!)        │
                              ┌─────▼─────┐
                              │ BP Decoder │  (30 iterations)
                              └─────┬─────┘
                                    │
                          ┌─────────┼─────────┐
                          │                   │
                     BP Success          BP Failed
                          │                   │
                          ▼                   ▼
                    🔵 CORRECTED_BP    ┌─────────────┐
                                       │  ORBGRAND   │
                                       └──────┬──────┘
                                              │
                                    ┌─────────┼─────────┐
                                    │                   │
                               ORBGRAND OK        ORBGRAND Fail
                                    │                   │
                                    ▼                   ▼
                              🟠 CORRECTED      ❌ UNRECOVERABLE

### BP Decoder — How it works:

1. Builds a parity-check matrix H from the CRC polynomial (r × n binary matrix)

2. Runs iterative message-passing on the Tanner graph (sum-product algorithm)

3. Check-to-variable update: each check node computes 2·atanh(∏ tanh(msg/2))

4. Variable-to-check update: each variable sums channel LLR + all incoming check messages

5. After each iteration, performs hard decision and checks CRC — if pass, terminates early

6. Maximum 30 iterations with 0.5 damping factor for convergence on dense CRC matrices

### ORBGRAND Decoder — How it works (if BP fails):

1. Sorts all bit positions by reliability: least reliable first (using |LLR| values)

2. Weight 1: tries flipping each of the least reliable bits, one at a time — checks CRC

3. Weight 2: tries flipping pairs of least reliable bits — checks CRC after each flip

4. Weight 3, 4, 5: continues with increasing combinations

5. For each candidate: if CRC passes → declares success and outputs corrected codeword

6. Budget: 5,000 queries (normal/good channel) or 15,000 queries (bad channel — more attempts)

| Error Weight | Correction Rate | Avg Queries Used |
| --- | --- | --- |
| 1 bit | ~95% | ~5 |
| 2 bits | ~80% | ~50 |
| 3 bits | ~60% | ~500 |
| 4 bits | ~35% | ~3,000 |
| 5 bits | ~15% | ~8,000 |

## Step 2.9 — Traffic Light Assignment

| Condition | Traffic Light | Meaning |
| --- | --- | --- |
| Aliasing injection or undetected error | 🔴 RED | Data integrity compromised |
| Corrected by BP or ORBGRAND | 🟠 ORANGE | Error detected and recovered |
| Unrecoverable error | 🔴 RED | Error too severe to correct |
| Clean pass (no errors) | 🟢 GREEN | Data integrity verified |

## Step 2.10 — Store Everything for Dashboard

All results are appended to session state history lists:

decision_log.append({frame_id, timestamp, channel_state, alias_prob, crc_mode, ...})
reliability_history.append(reliability)      # For the Reliability line chart
overhead_history.append(overhead_base)        # For the Overhead line chart
aliasing_prob_history.append(alias_prob)      # For the Threshold timeline
crc_mode_timeline.append(crc_mode)            # For protocol tracking

# Phase 3: Dashboard Rendering

After process_single_frame() returns, Streamlit re-renders the entire page with updated data:

## 3.1 — Status Cards (top row)

| Card | Shows | Source |
| --- | --- | --- |
| Active Polynomial | CRC-16 / CRC-32 / Checksum badge | latest["crc_mode"] |
| ORBGRAND Depth | 5,000 or 15,000 | latest["orb_queries"] |
| Channel State | GOOD (green) or BAD (red) | latest["channel_state"] |
| System Status | Traffic light (3 glowing circles) | current_traffic_light |

## 3.2 — Statistics Row

| Metric | Calculation |
| --- | --- |
| Total Frames | stats["total"] |
| CRC-32 Upgrades | Count and percentage of upgrade decisions |
| Energy Saves | Count and percentage of Checksum usage |
| Recoveries | BP corrections + ORBGRAND corrections |
| Avg Reliability | mean(reliability_history) |

## 3.3 — Decision Log

A scrollable container showing the last 40 entries, each with emoji icon, frame number, timestamp, result status, and the controller's reasoning.

🟢 #001 02:05:04 [PASS]          — ✅ Standard CRC-16 (aliasing prob = 0.41%)
🟠 #008 02:05:06 [CORRECTED_ORB]  — ✅ Standard CRC-16 | 🔄 ORBGRAND ↑ 5,000
❌ #015 02:05:08 [UNRECOVERABLE]  — ⚠️ Aliasing Prob 99.5% → UPGRADING to CRC-32

## 3.4 — Plotly Charts

### Chart 1: Reliability vs. Computational Overhead

Dual Y-axis Plotly line chart. Left axis: Reliability score (0 to 1) as teal line with area fill. Right axis: Overhead cost as orange dashed line. X-axis: Frame number.

### Chart 2: XGBoost Aliasing Probability Gauge

Semicircular Plotly gauge from 0% to 100%. Green zone (0-15%), Yellow zone (15-50%), Red zone (50-100%). Shows current aliasing probability and delta from previous frame.

### Chart 3: Aliasing Probability Over Time

Time series: blue line = XGBoost's prediction per frame; orange dashed = 15% threshold; red shaded regions = frames where system upgraded to CRC-32.

## 3.5 — Correction Log Table

Shows frames where BP or ORBGRAND successfully corrected errors. Displays frame number, method used, error count, first 32 bits of original, first 32 bits of recovered, and whether they match.

# What Makes This 'Intelligent'?

The key difference from a textbook CRC system is the feedback loop:

| Aspect | Traditional CRC | Our Deployed Model |
| --- | --- | --- |
| Polynomial | Fixed (always CRC-16) | Dynamically switches CRC-16 / CRC-32 / Checksum |
| Prediction | No prediction | XGBoost predicts danger BEFORE it happens |
| Error handling | Detect only, discard on error | BP + ORBGRAND attempt live correction |
| Parameters | Fixed query depth | ORBGRAND adapts: 5K ↔ 15K queries |
| Channel awareness | None | Gilbert-Elliott monitor tracks channel health |
| User visibility | None | Live dashboard with every decision + reasoning |

IMPORTANT: The XGBoost model is the decision trigger — it runs on every single frame, evaluates 14 features in microseconds, and tells the controller: 'There's a 22% chance this frame's error will slip through CRC-16 undetected. Upgrade now.'

# Summary: One Frame in 10 Steps

| Step | What Happens | Key Component |
| --- | --- | --- |
| 1 | User sets BER, channel state, payload | Sidebar widgets |
| 2 | Estimate noise conditions (type, errors, burst) | process_single_frame() |
| 3 | XGBoost predicts aliasing probability | AliasingPredictor.predict_aliasing_probability() |
| 4 | Controller decides: CRC-16 / CRC-32 / Checksum | Switching logic (if/elif/else) |
| 5 | Controller decides: ORBGRAND 5K or 15K | Channel state check |
| 6 | CRC encode the message | CRCEncoder.encode() |
| 7 | Transmit through Gilbert-Elliott channel | ChannelModel.gilbert_elliott_transmit() |
| 8 | CRC verify → if fail → BP → ORBGRAND | BPDecoder + ORBGRANDDecoder |
| 9 | Set traffic light 🟢 🟠 🔴 | Traffic light assignment |
| 10 | Log everything, update charts | Session state + Plotly re-render |

M.Tech Computer Networks — Streamlit Model Working Explained